Ethernet PHY
Information Model

Version 1.0.0-1,

16th of February 2019

ONF TR‑541

ONF Document Type: Technical Recommendation

ONF Document Name: Ethernet PHY Information Model Version 1.0

# Disclaimer

THIS SPECIFICATION IS PROVIDED "AS IS" WITH NO WARRANTIES WHATSOEVER, INCLUDING ANY WARRANTY OF MERCHANTABILITY, NONINFRINGEMENT, FITNESS FOR ANY PARTICULAR PURPOSE, OR ANY WARRANTY OTHERWISE ARISING OUT OF ANY PROPOSAL, SPECIFICATION OR SAMPLE.

Any marks and brands contained herein are the property of their respective owners.

# Open Networking Foundation

1000 El Camino Real, Suite 100, Menlo Park, CA 94025

www.opennetworking.org

©2017 Open Networking Foundation. All rights reserved.

Open Networking Foundation, the ONF symbol, and OpenFlow are registered trademarks of the Open Networking Foundation, in the United States and/or in other countries. All other brands, products, or service names are or may be trademarks or service marks of, and are used to identify, products or services of their respective owners.

# Table of Contents

Disclaimer	2

Open Networking Foundation	2

Table of Contents	3

List of Figures	5

List of Tables	5

Document History	6

1	Introduction	6

2	Definitions	6

2.1	Terms	6

2.2	Abbreviations and Acronyms	6

3	Compliance Statement	7

4	Overview of the Ethernet PHY Information Model	7

4.1	Coverage	7

4.2	Overview	8

5	Conventions	10

5.1	UML Modeling Conventions	10

5.2	Default Values	10

5.3	Comments	11

5.4	ONF Stereotypes	11

5.4.1	attributeValueChangeNotification	11

5.4.2	objectCreationNotification and objectDeletionNotification	11

5.4.3	isInvariant	11

5.4.4	valueRange	12

5.4.5	partOfObjectKey	12

5.4.6	unit	12

5.4.7	Support Qualifier	13

5.4.8	bitLength	13

5.4.9	Unsigned, Encoding and Counter	13

6	Special elements	13

6.1	PMD Definitions	13

6.2	Shared *_Pacs	13

6.3	Problem Definitions, Associations and Inheritances	13

6.4	Performance Values	13

7	Attachment to the Core Information Model	14

7.1	Attachment of *_Pacs	16

8	Non-Backward Compatibility of the next Version	16

9	Future Work	17

10	UML Model Files	17

11	YANG Model Files	17

12	Interface Simulator	18

13	Data Dictionary	18

13.1	WireInterface_Pac	19

13.1.1	WireInterface_Pac	20

13.1.2	WireInterfaceCapability	22

13.1.3	WireInterfaceConfiguration	28

13.1.4	WireInterfaceStatus	32

13.1.5	WireInterfaceCurrentProblems	35

13.1.6	WireInterfaceCurrentPerformance	36

13.1.7	WireInterfaceHistoricalPerformances	37

13.2	PureEthernetStructure_Pac	38

13.3	EthernetContainer_Pac	39

13.4	Data Types	40

13.4.1	PmdType	40

13.4.2	WireInterfaceProblemSeverityType	42

13.4.3	WireInterfaceCurrentProblemType	43

13.4.4	WireInterfacePerformanceType	43

13.4.5	WireInterfaceCurrentPerformanceType	45

13.4.6	WireInterfaceHistoricalPerformanceType	45

13.5	Enumeration Types	46

13.5.1	LoopBackType	46

13.5.2	SeverityType	46

13.5.3	MdiKindType	46

13.5.4	MediumKindType	47

13.5.5	MiiKindType	48

13.5.6	DuplexType	49

13.5.7	SignalOrderingType	49

13.5.8	RxSyncPreferenceType	49

13.5.9	RxSyncRoleType	50

13.5.10	pmdNegotiationStateType	50

13.6	Super Classes	51

13.6.1	WireCurrentProblem	51

13.7	Notifications	52

13.7.1	AttributeValueChangedNotification	52

13.7.2	ObjectCreationNotification	54

13.7.3	ObjectDeletionNotification	55

13.7.4	ProblemNotification	56

14	References	58

15	Back matter	58

15.1	Editors	58

15.2	Contributors	58

# List of Figures

Figure 1: Model Overview	9

Figure 2: Example for the sub-structuring of *_Pacs	10

Figure 3: Usage of ONF Core Information Model 1.2	15

Figure 4: Associations between LayerProtocol and *_Pac	16

Figure 5: WireInterface_Pac	19

Figure 6: Exemplary PureEthernetStructure_Pac	38

Figure 7: Exemplary EthernetContainer_Pac	39

# List of Tables

Table 1: Attributes for WireInterface_Pac	20

Table 2: Attributes for WireInterfaceCapability	22

Table 3: Attributes for WireInterfaceConfiguration	28

Table 4: Attributes for WireInterfaceStatus	32

Table 5: Attributes for WireInterfaceCurrentProblems	36

Table 6: Attributes for WireInterfaceCurrentPerformance	36

Table 7: Attributes for WireInterfaceHistoricalPerformances	37

Table 8: Attributes for PmdType	40

Table 9: Attributes for WireInterfaceProblemSeverityType	42

Table 10: Attributes for WireInterfaceCurrentProblemType	43

Table 11: Attributes for WireInterfacePerformanceType	43

Table 12: Attributes for WireInterfaceCurrentPerformanceType	45

Table 13: Attributes for WireInterfaceHistoricalPerformanceType	45

Table 14: Attributes for WireCurrentProblem	51

Table 15: Attributes for AttributeValueChangedNotification	53

Table 16: Attributes for ObjectCreationNotification	54

Table 17: Attributes for ObjectDeletionNotification	55

Table 18: Attributes for ProblemNotification	56

# Document History

| Version | Date | Description of Change |
| --- | --- | --- |
| 0.0 | 2017-12-09 | Initial version of the Ethernet PHY Information Model as technology specific extension to the TR-512 ONF Core Information Model [ONF Core IM] |
| 0.1 | 2019-02-16 | Consolidating all improvements discussed after first implementation and testing during the 5th ONF PoC; provided for review on project level. |

# Introduction

This ONF Technical Recommendation (ONF TR‑541) describes a management interface for vendor agnostic management of the Ethernet OSI Layer 1.

It is mainly based on IEEE 802.3 Standard for Ethernet [IEEE802.3], but consolidates also elements of specifications provided by the SFF Committee (now a Technology Affiliate of the Storage Networking Industry Association) and other sources.

The comprised UML information model represents a technology specific extension to the TR-512 ONF Core Information Model 1.2 [ONF Core IM] and it integrates well with the ONF TR-532v1.1 Microwave Information Model [ONF MW IM] for providing a unified Ethernet transport.

# Definitions

## Terms

The primary purpose of this document is to define terms and hence terms are defined throughout the document. Key terms are highlighted in section 2.2 Abbreviations and Acronyms.

## Abbreviations and Acronyms

| Term | Explanation |
| --- | --- |
| CWDM | Coarse Wavelength Division Multiplex |
| EFM | Ethernet First Mile |
| EPON | Ethernet Passive Optical Network |
| MAU | Medium Attachment Unit |
| ONF | Open Networking Foundation |
| OSI | Open Systems Interconnection Model |
| *_Pac | Technology Specific Conditional Package |
| PHY | Physical Layer |
| PMD | Physical Medium Dependent |
| SDN | Software Defined Network |
| SFF | Small Form Factor |
| TR | ONF Technical Recommendation |

# Compliance Statement

Completeness

A device's interface must not be denoted as compliant to this Ethernet PHY Information Model, if it doesn't implement all components.

Support

The hardware does not necessarily need to make available all functionalities covered by this modeling. In case some functionality is not available at the hardware, the device's interface shall answer the default values defined in this model. All functionalities, which are available at the hardware and covered by the Ethernet PHY Information Model, must be manageable with this model (to the extent of the comprised attributes).

Proprietary Extensions

Interfaces implementing elements of this Ethernet PHY Information Model plus additional components must not be denoted as compliant. Such interfaces might be called "based on" or "expanding" it.

# Overview of the Ethernet PHY Information Model

## Coverage

Basically, the structure of the Ethernet PHY Information Model is capable of covering all kinds of point-to-point Physical Medium Dependent (PMD).

The current set of attributes allows managing at least the following PMDs:

| 1BASE5 | 100BASE-T4 | 1000BASE-BX10 | 10GBASE-LX4 | 40GBASE-KR4 | 100GBASE-CR4 |
| --- | --- | --- | --- | --- | --- |
| 10BASE2 | 100BASE-TX | 1000BASE-LX | 10GBASE-CX4 | 40GBASE-CR4 | 100GBASE-KR4 |
| 10BROAD36 | 100BASE-BX10 | 1000BASE-LX10 | 10GBASE-KX4 | 40GBASE-SR4 | 100GBASE-KP4 |
| 10BASE-T | 100BASE-FX | 1000BASE-SX | 10GBASE-ER | 40GBASE-LR4 | 100GBASE-CR10 |
| 10BASE-FP | 100BASE-LX10 | 1000BASE-CX | 10GBASE-LR | 40GBASE-ER4 | 100GBASE-SR4 |
| 10BASE-FB | 100BASE-T2 | 1000BASE-KX | 10GBASE-SR | 40GBASE-FR | 100GBASE-SR10 |
| 10BASE-FL |  | 1000BASE-T | 10GBASE-LRM |  | 100GBASE-LR4 |
|  |  |  | 10GBASE-KR |  | 100GBASE-ER4 |
|  |  |  | 10GBASE-PR |  |  |

In principle, the Ethernet PHY Information Model allows definition of additional PMDs during implementation, but very sophisticated, highest capacity PMDs might require additional attributes, which are not yet included.

The Ethernet PHY Information Model does not cover any components of the Ethernet MAC layer.

The following PMDs are not yet cover, but might become subject to future expansion, if requested:

Point-to-multipoint Ethernet for Ethernet Passive Optical Network (EPON)
like 1000BASE-PX and 10/1GBASE-PRX

PMDs with variable line speed for Ethernet First Mile (EFM)
like 2BASE-TL and 10PASS-TS

PMDs for Ethernet over SDH
like 10GBASE-W

The Ethernet PHY Information Model provides the necessary attributes for

the device informing the SDN Controller about its capabilities

the Controller configuring the device

the device providing status, problem and performance information

## Overview

The Ethernet PHY Information Model consists of the following three layers, which are represented by *_Pacs (see also chapter 7.1 Attachment of *_Pacs):

Interface

The WireInterface_Pac contains the characteristics of the physical layer of the wire based Ethernet interface. It represents the combination of the interface at the internal Ethernet switch and the "transmitter" (e.g. SFP). It is specified within this document.

Structure

Basic purpose of the Structure layer is segmenting the physical resource provided by the Interface layer into logical pieces that can be booked by Containers. In case of nowadays wire based Ethernet interfaces, just a single segment is provided. The Ethernet PHY Information Model is not defining any new Structure, but referencing on the PureEthernetStructure_Pac defined in the ONF TR-532v1.1 Microwave Information Model [ONF MW IM].

Container

The Container layer offers transport services to higher protocol layers. It represents the upper demarcation of the modeling. The Ethernet PHY Information Model is not defining any new Container, but referencing on the EthernetContainer_Pac defined within the ONF TR-532v1.1 Microwave Information Model [ONF MW IM].

The following picture shall help understanding the layering of the Technology Specific Conditional Packages (*_Pacs), which are shared between ONF TR-532v1.1 Microwave Information Model [ONF MW IM] and ONF TR‑541‑v1.0 Ethernet PHY Information Model. (It has to be noted that this picture is for explanatory purposes only. The associations between the *_Pacs are implemented in a more complex way, which is described in chapter 7.1 Attachment of *_Pacs.)

Figure 1: Model Overview

All Technology Specific Conditional Package (*_Pacs) are sub-structured into the following classes:

Capabilities – The device informs about its features, characteristics and tuning ranges

Configuration – The Controller configures the device and the device informs about its current configuration

Status – The device informs about measurement values and its current operational status

Problems – The device proactively informs about events

Current Performance – The device informs about the current status of its performance counters

Historical Performance – The device informs about the status of its performance counters at the end of a well-defined time period

Figure 2: Example for the sub-structuring of *_Pacs

Each Technology Specific Conditional Package (*_Pac) is sufficiently described by one set of capability, configuration, status and current performance attributes, but several current problem and historical performance value sets might be required. Nevertheless, *CurrentProblems and *HistoricalPerformances classes are in a one to one association to the *_Pac. The necessary multiplicity is generated by lists inside these classes. This additional hierarchy is required by the mediator implementation after conversion to YANG.

# Conventions

For seamless integration of components of this ONF TR‑541‑v1.0 Ethernet PHY Information Model and ONF TR-532v1.1 Microwave Information Model [ONF MW IM] both models are following the same conventions.

As a consequence, this chapter about conventions is identical with chapter 5 Conventions in ONF TR-532v1.1, except the exact content of the unit table in chapter 5.4.6 unit. Because ONF TR‑541‑v1.0 does not contain alternative Structure_Pacs or alternative Container_Pacs, chapter 5.4.7 Support Qualifier differs from ONF TR-532v1.1, too.

## UML Modeling Conventions

This TR follows the conventions as described in the TR-512 ONF Core Information Model 1.2 [ONF Core IM] and the TR-514 ONF UML Modeling Guidelines [ONF UMGL].

## Default Values

The default values in the Ethernet PHY Information Model have been defined in accordance to the following basic principles:

Every attribute (except keys, which have to be unique) shall have a default value.

The default value shall be inside the value range of the data type of the attribute.

Capability attributes:

The default value shall either indicate unavailability of the functionality (if applicable)

or be outside the range of reasonable values of the attribute.

Configuration, status and performance attributes:

The default value shall either represent the configuration, status or performance measurement value right after starting the device (in case such a "neutral" value is applicable to the attribute)

or be outside the range of reasonable values of the attribute.

Lists of data types shall contain the minimum multiplicity number of elements. This also means that lists of data types with a minimum multiplicity of zero shall have "null" as default value.

## Comments

The comprised comments are meant to explain the attributes in such a way that further documentation is not required for understanding the attributes' meaning.

In rare cases (e.g. Problems) the modeling is open for amending device specific elements. This is done by Capability attributes of data type String, which are foreseen to contain an unspecified number of elements separated by comma. In such cases, the comment field is also used for defining a minimum set of elements, which must be supported. Of course, adding additional elements to these lists is not seen as a Proprietary Extensions according to chapter 3.

Double quotes have been avoided within the comments for supporting the conversion to YANG.

## ONF Stereotypes

### attributeValueChangeNotification

This stereotype defines whether a notification has to be raised, when the attribute changes its value.

It has been set on "true" for all attributes, which are comprised in Configuration classes or in data types used by attributes of the Configuration classes.

It has also been set on "true" for status attributes, which might be subject to automated changes, but do not represent gradually changing measurement values.

The attributeValueChangeNotification stereotype has been set on "false" for status attributes, which are exclusively following configuration activities. This is for avoiding double messaging.

### objectCreationNotification and objectDeletionNotification

These stereotypes define whether a notification has to be raised when an instance of a class has been created, respectively deleted.

It has been set on "true" for all *_Pac classes, which are potentially attached to the LayerProtocol class of the Core Information Model (see chapter 7.1 Attachment of *_Pacs for details).

### isInvariant

This stereotype defines whether the value of the attribute can be changed, or not, after it has been created.

It has been set on "true" (means: cannot be changed) for the following attributes:

*Ids, which are representing target addressed for referencing data types or classes, (except ContainerIDs, which are required for connecting logical traffic from outside the reach of the modeling with a Container)

All attributes, which are comprised in Capability classes or in data types used by attributes of the Capability classes

All attributes, which are comprised in data types that are attached to the CurrentProblems classes.

All attributes, which are comprised in data types that are attached to the *Performance classes.

All attributes, which are comprised in Notifications

It has been set on "false" (means: can be changed) for the following attributes:

All attributes, which are comprised in Configuration classes

All attributes, which are comprised in Status classes or in data types used by attributes of the Status classes

### valueRange

This stereotype identifies the allowed values for the attribute.

It has not been used within the Ethernet PHY Information Model, because of a conflict with the policy defined for default values in chapter 5.2

### partOfObjectKey

This stereotype indicates whether the attribute is part of the object key or not. Value "0" (default) means the attribute is not part of the object key. Values > "0" indicate that the attribute is part of the object key and the value defines the order of the attribute in case the key is composed of more than one attribute.

The partOfObjectKey stereotype has been set on "1" for all *Id attributes, which are of data type UniversalId, but not the *IdRef attributes, which are referring on them.

It does also not apply on containerID attributes that represent a configurable string and are required for associating traffic flows to the transport resources (Container) provided by the Ethernet PHY Information Model.

### unit

This optional stereotype contains a textual definition of the unit associated with the attribute's value.

The following units have been used in the Ethernet PHY Information Model

| Unit | Meaning |
| --- | --- |
| % | Percentage |
| Bytes | Total number of Bytes |
| Bytes/min | Bytes per minute |
| Bytes/s | Bytes per second |
| Celsius | Degree Celsius |
| dB | Decibel |
| dBm | Decibel milliwatt |
| kbit/s | 1000 bit per second |
| kHz | 1000 Hertz |
| m | Meter |
| mJ | Millijoule |
| ms | Millisecond |
| mW | Milliwatt |
| octets | Total number of Octets |
| pm | Picometer |
| ps | Picosecond |
| s | Second |
| symbols | Number of symbols |

### Support Qualifier

The SupportQualifier has always been left on its default value "mandatory":

### bitLength

The bitLength stereotype has been used to define Integer primitives smaller than 64bit.

### Unsigned, Encoding and Counter

The unsigned, encoding and counter stereotypes have not been applied.

# Special elements

## PMD Definitions

PMDs are defined by name, speed and duplex type in a separated data type in the Capability section of the Ethernet PHY Information Model. The comment field of the PmdType::pmdName attribute prescribes a list of PMD names (according to [IEEE802.3]). During interface implementation, the supported PMDs have to be named according to this prescription. Only in case additional PMDs, which are not listed in the comment, would be defined, additional names could be applied. In case such additional PMD would be described in any future version of IEEE 802.3 Ethernet [IEEE802.3], the name stated there shall be applied.

## Shared *_Pacs

The PureEthernetStructure_Pac of this ONF TR‑541‑v1.0 Ethernet PHY Information Model is equivalent to the PureEthernetStructure_Pac of the ONF TR-532v1.1 Microwave Information Model [ONF MW IM].

The EthernetContainer_Pac of this ONF TR‑541‑v1.0 is equivalent to the EthernetContainer_Pac of the ONF TR-532v1.1.

## Problem Definitions, Associations and Inheritances

The problem definitions, associations and inheritances are identical to the definitions made ONF TR-532v1.1 (see chapter 6.3 of [ONF MW IM]).

## Performance Values

The performance value definitions, associations and inheritances are identical to the definitions made ONF TR-532v1.1 (see chapter 6.4 of [ONF MW IM]).

# Attachment to the Core Information Model

The Core Information Model is the basis of the Ethernet PHY Information Model. However not all object classes of the Core Information Model are necessary for a control interface between Devices and SDN Controller. For example the root element of the Ethernet PHY Information Model is the NetworkElement object class. The SdnController object class must not be implemented in network elements, because a network element cannot offer any control for SDN Controllers.

The following classes of the Core Information Model are not used by the Ethernet PHY Information Model:

SdnController

NetworkControlDomain

Link

LinkPort

FcRoute

Figure 3 highlights the classes of the Core Information Model, which are part of the Ethernet PHY Information Model.

Figure 3: Usage of ONF Core Information Model 1.2

The LogicalTerminationPoints (LTP) is the most important class. Its associations allow complex hierarchies of LTPs. A LayerProtocol class provides content to the generic LTP class and describes its function.

The following layerProtocolName values have to be applied:

| layerProtocolName | Type of *_Pac |
| --- | --- |
| ETC | EthernetContainer |
| MWS | PureEthernetStructure |
| PHYPS | WireInterface |

## Attachment of *_Pacs

Technology specific extensions of the Core Information Model are implemented by extending the LayerProtocol class with conditional packages. These conditional packages are called *_Pacs.

The Ethernet PHY Information Model defines three technology specific conditional packages, pointing to the LayerProtocol:

The WireInterface_Pac defines an LTP as Ethernet PHY section trail termination point (PHYPS-TTP).

The PureEthernetStructure_Pac defines an LTP as Ethernet PHY and microwave section trail termination point (MWS-TTP) for pure Ethernet transport systems.

The EthernetContainter_Pac defines an LTP as a Ethernet PHY and microwave client connection termination point (MW-Client-CTP) for an Ethernet client signal.

Figure 4 shows the associations between the LayerProtocol class of the Core Information Model and the three microwave specific conditional packages. Please note that for simplicity reasons the ForwardingConstruct class is not displayed.

Figure 4: Associations between LayerProtocol and *_Pac

# Non-Backward Compatibility of the next Version

ONF TR‑541‑v1.0 comprises not just the technology specific *_Pacs for managing the Ethernet PHY interface, but also elements of the ONF Core Information Model. Because the next version of ONF TR‑541 will require to comprise an updated version of the Core Information Model, which is not backward compatible, also the next version of ONF TR‑541 will not be backward compatible.

Please, see further information about future work, which will also impact backward compatibility, in chapter 9.

# Future Work

It is seen as a problem that ONF TR-532 and ONF TR‑541 are not disjunctive in regards to pureEthernetStructure_Pac and EthernetContainer_Pac. It is considered to decompose both information models into *_Pacs, make these *_Pacs available in separate repositories, and to just reference these *_Pacs inside the respective TR documents. Apart from avoiding redundancy, this decomposition would also allow updating the *_Pacs independently from each other. As a consequence, each *_Pac would be delivered as a separate YANG file.

Current version of ONF TR‑541 is limited on technology specific conditional packages to the LayerProtocol class of the Core Information Model. This results in a comprehensive model for managing devices. From the controller's and the applications' points of view, these devices have to be connected to describe a network topology. The necessary classes for describing such topologies are also comprised in the Core Information Model, but they require technology specific conditional packages in the same way the LayerProtocol class does. It is considered to define Ethernet PHY specific conditional packages also for the ForwardingConstruct and to reference them in the next version of ONF TR‑541.

The next version of ONF TR‑541 shall also take benefit from the more advanced modeling of equipment, connectors and topology in later versions of the Core Information Model. E.g. several attributes, which are detailing characteristics of the SFP, will be replaced to equipment related classes then.

# UML Model Files

The Papyrus export, which is holding the UML modeling files of the ONF TR‑541‑v1.0 Ethernet PHY Information Model can be found on

https://github.com/openBackhaul/wireInterface/tree/TR541v1

The available .zip file comprises the

WireInterface_Pac

and all related datatypes, notifications and imported artefacts.

Please note that YANG Model files, Data Dictionary (GenDoc export) and Interface Simulator are just implementations of the UML modeling files.

In case of any divergence, the UML modeling files are always the relevant data base.

# YANG Model Files

The YANG files generated from the UML model referenced in chapter 10 can be found on

not yet generated

# Interface Simulator

An Interface Simulator is for supporting application development without own hardware. The Interface Simulator, which is implementing the UML model referenced in chapter 10 can be found on

not yet generated

# Data Dictionary

Please be aware that the following lists are showing not all the attribute's characteristics and stereotypes, e.g. default value and unit are missing.

## WireInterface_Pac

Figure 5: WireInterface_Pac

### WireInterface_Pac

Qualified Name: WireModel::ObjectClasses::WireInterface::WireInterface_Pac

Applied stereotypes:

- OpenModelClass

- objectCreationNotification: YES

- objectDeletionNotification: YES

- support: MANDATORY

Table 1: Attributes for WireInterface_Pac

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| _layerProtocol | LayerProtocol ./. | 1 | RW | OpenModelAttribute - partOfObjectKey: 1 - AVC: NO - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | CoreModel-CoreNetworkModule-ObjectClasses:NetworkElement/_ltpRefList/_lpList/uuid |
| _wirebasedinterfacecapability | WireInterfaceCapability ./. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: No - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | See referenced class |
| _wirebasedinterfaceconfiguration | WireInterfaceConfiguration ./. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: No - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | See referenced class |
| _wirebasedinterfacestatus | WireInterfaceStatus ./. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: No - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | See referenced class |
| _wirebasedinterfacecurrentproblems | WireInterfaceCurrentProblems ./. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: No - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | See referenced class |
| _wirebasedinterfacecurrentperformance | WireInterfaceCurrentPerformance ./. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: No - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | See referenced class |
| _wirebasedinterfacehistoricalperformances | WireInterfaceHistoricalPerformances ./. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: No - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | See referenced class |

### WireInterfaceCapability

Qualified Name: WireModel::ObjectClasses::WireInterface::WireInterfaceCapability

Applied stereotypes:

- OpenModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- support: MANDATORY

Table 2: Attributes for WireInterfaceCapability

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| supportedPmdList | PmdType ./. | 1..* | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | List of Physical Medium Dependent (PMD) that can be operated |
| autoPmdNegotiationIsAvail | Boolean false | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | 1 = Indicates that device is supporting Auto-negotiation |
| autoNegotiationPmdSelectionIsAvail | Boolean false | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | 1 = device is supporting restricting auto negotiation on a pre-defined list of PMDs |
| signalOrderingList | SignalOrderingType ./. | 1..* | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Describes the different (e.g. MDI, MDI-X) ways of ordering the signals on the physical medium |
| autoSignalOrderingIsAvail | Boolean false | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | 1 = there is a mechanism for automatically crossing over tx and rx implemented |
| miiKind | MiiKindType NOT_YET_DEFINED | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Kind of Medium Independent Interface (MII) provided by this Medium Attachment Unit (MAU) (e.g. SFP, moldered port) |
| mdiKind | MdiKindType NOT_YET_DEFINED | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Kind of Medium Dependent Interface (MDI) provided by this Medium Attachment Unit (MAU) |
| requiredMediumKind | MediumKindType NOT_YET_DEFINED | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Kind of medium required for operating this Medium Attachment Unit (MAU), more like an information field |
| wavelengthMin | Integer -1 | 0..3 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: LENGTH_32_BIT - unit: pm - support: MANDATORY | Source: SFF-8690. Minimum laser wavelength in pico meter, -1 = not applicable, 0 = not known, wavelengthMax = wavelength cannot be configured; multiplicity=0..3 for 10GBASE-LX4 according to 802.3 53.5 |
| wavelengthMax | Integer -1 | 0..3 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: LENGTH_32_BIT - unit: pm - support: MANDATORY | Source: SFF-8690. Maximum laser wavelength in pico meter, -1 = not applicable, 0 = not known, wavelengthMin = wavelength cannot be configured; multiplicity=0..3 for 10GBASE-LX4 according to 802.3 53.5 |
| wavelengthGridMin | Integer -1 | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: LENGTH_32_BIT - unit: pm - support: MANDATORY | Source: SFF-8690. Minimum grid spacing supported by the transceiver, -1 = not applicable, 0 = not known |
| linkLengthMax | Integer -1 | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: LENGTH_32_BIT - unit: m - support: MANDATORY | Indicates the maximum link length that is supported by the transceiver on the medium, which is specified in the standard referenced in TypeDefinitions::phyType::phyKind. |
| vendorName | String NOT_YET_DEFINED | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY LikelyToChange | Name of the vendor of the transceiver Might be moved to CoreModel::CorePhysicalModel-Initial::EquipmentDetail::ObjectClasses::InvariantDetails::ManufacturerProperties::manufacturerName |
| vendorOui | String NOT_YET_DEFINED | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY LikelyToChange | Describes the IEEE Company identifier of the vendor of the transceiver (1st part of 802.3 ResourceTypeID) 802.3 22.2.4.3.1 PHY Identifier; also referenced in 45.2.1.13 PMA/PMD package identifier Might be moved to CoreModel::CorePhysicalModel-Initial::EquipmentDetail::ObjectClasses::InvariantDetails::ManufacturerProperties::manufacturerIdentifier |
| partNumber | String NOT_YET_DEFINED | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY LikelyToChange | Uniquely identifies the transceiver in the vendor's product lists 802.3 22.2.4.3.1 PHY Identifier; also referenced in 45.2.1.13 PMA/PMD package identifier as six bit model number Might be moved to CoreModel::CorePhysicalModel-Initial::EquipmentDetail::ObjectClasses::InvariantDetails::EquipmentType::partTypeIdentifier |
| revisionNumber | String NOT_YET_DEFINED | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY LikelyToChange | Identifies the revision number of the transceiver (3rd part of 802.3 ResourceTypeID) Might be moved to CoreModel::CorePhysicalModel-Initial::EquipmentDetail::ObjectClasses::InvariantDetails::EquipmentType::version 802.3 22.2.4.3.1 PHY Identifier; also referenced in 45.2.1.13 PMA/PMD package identifier as four-bit revision number |
| serialNumber | String NOT_YET_DEFINED | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY LikelyToChange | Vendor's serial number for the transceiver. 0 = not applicable Might be moved to CoreModel::CorePhysicalModel-Initial::EquipmentDetail::ObjectClasses::InvariantDetails::EquipmentInstance::serialNumber |
| dateCode | String NOT_YET_DEFINED | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY LikelyToChange | Vendor's date code for the transceiver Might be moved to CoreModel::CorePhysicalModel-Initial::EquipmentDetail::ObjectClasses::InvariantDetails::EquipmentInstance::manufactureDate |
| shortReachModeIsAvail | Boolean false | 1 | R | Experimental OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | 1 = Indicates that Short Reach Mode for 10GBASE-T according to 802.3 45.2.1.64 is available |
| eeeIsAvail | Boolean false | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | 1 = Indicates that Energy-Efficient Ethernet (EEE) is available at the device. |
| unidirectionalOperationIsAvail | Boolean false | 1 | R | Experimental OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | 1 = Medium Attachment Unit (MAU) able to transmit from Media Independent Interface (MII) regardless of whether the MAU has determined that a valid link has been established, 0 = MAU able to transmit from MII only when the MAU has determined that a valid link has been established 802.3 |
| rxlevelLowThreshold | Integer 99 | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: LENGTH_8_BIT - unit: dBm - support: MANDATORY | Threshold for alarming low RX levels. Value pre-defined by SFP manufacturer (SFF- 8472) |
| rxlevelHighThreshold | Integer 99 | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: LENGTH_8_BIT - unit: dBm - support: MANDATORY | Threshold for alarming high RX levels. Value pre-defined by SFP manufacturer (SFF- 8472) |
| temperatureLowThreshold | Integer -99 | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: LENGTH_8_BIT - unit: Celsius - support: MANDATORY LikelyToChange | Might move to somewhere in the Physical Segment of the Core IM Threshold for alarming low temperature values. Value pre-defined by SFP manufacturer (SFF- 8472) |
| temperatureHighThreshold | Integer -99 | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: LENGTH_8_BIT - unit: Celsius - support: MANDATORY LikelyToChange | Threshold for alarming high temperature values. Value pre-defined by SFP manufacturer (SFF- 8472) Might move to somewhere in the Physical Segment of the Core IM |
| supportedAlarmList | String NOT_YET_DEFINED | 6..* | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Available alarms to be listed. Mandatory:'txFault', 'rxLos', 'tempHigh', 'tempLow', 'rxLevelHigh', 'rxLevelLow'. Optional:'vccHigh', 'vccLow', 'txBiasHigh', 'txBiasLow', 'txPowerHigh', 'txPowerLow', 'laserTempHigh', 'laserTempLow', 'tecCurrentHigh', 'tecCurrentLow'. Further alarms might be added by the device. MW IM |
| supportedLoopBackKindList | LoopBackType NONE | 1..5 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Source: 802.3 45.2.1.12.1 PMA remote loopback ability. List of supported kinds of looping back of header information to the remote site. |
| maintenanceTimerRange | String Range of the maintenance timer not yet defined. | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: s - support: MANDATORY | Available time periods for maintenance configurations (e.g. the loop back) to be described. Concrete values shall be separated by commas (e.g. '10, 60, 360'). Ranges shall be expressed as two values separated by a minus (e.g. '10-360'). |

### WireInterfaceConfiguration

Qualified Name: WireModel::ObjectClasses::WireInterface::WireInterfaceConfiguration

Applied stereotypes:

- OpenModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- support: MANDATORY

Table 3: Attributes for WireInterfaceConfiguration

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| wireInterfaceName | String Not yet defined. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Text field for the wire interface being named by the operator. Ideally used for entering unique numbers or names for unambiguously identifying the connection within the network |
| remoteWireInterfaceName | String Not yet defined. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Text field for defining the wire interface this one is connected with. Ideally used for entering unique numbers or names for unambiguously identifying the connection within the network |
| interfaceIsOn | Boolean true | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Source: 802.3 according 30.3.2.2.1 acPhyAdminControl. 1 = Activation of the interface (it gets powered and can be managed even if the transceiver is not yet transmitting or receiving). In case there is no Medium Attachment Unit (MAU) (e.g. no SFP in the cage) SETting (interfaceIsOn=1) must be ignored and GETing must return (interfaceIsOn=0) |
| transceiverIsOnList | Boolean false | 1..11 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Source 802.3 45.2.1.8 PMD transmit disable register (Register 1.9). 1 = Activation of the transmitter and receiver (e.g. laser) of the PHY; transceiverIsOnList[0]:total interface; transceiverIsOnList[1..10] different lanes of a multilane Medium Attachment Unit (MAU) |
| autoPmdNegotiationIsOn | Boolean false | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | 1 = Auto-negotiation is switched on |
| _fixedPmd | PmdType ./. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | If (autoPmdNegotiationIsOn=0) configuration of the concrete kind of Physical Medium Dependent (PMD). If (autoNegotiationIsOn=1) value of this field becomes irrelevant |
| _autoNegotiationPmdList | PmdType ./. | 0..* | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | If (autoPmdNegotiationIsOn=1) AND (autoNegotiationPmdSelectionIsAvail=1), this list defines the selection of PMDs the automated negotiation process is allowed to choose from. If no entry, the automated negotiation process is allowed to choose from all available PMDs. |
| autoSignalOrderingIsOn | Boolean false | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | 1 = e.g. auto-MDI-X is switched on |
| fixedSignalOrderingKind | SignalOrderingType NOT_YET_DEFINED | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | If (autoSignalOrderingIsOn=0) configuration of the concrete kind of signal ordering on the media (e.g. MDI, or MDI-X). If (autoSignalOrderingIsOn=1) value of this field becomes irrelevant |
| _rxSyncPreference | RxSyncPreferenceType NOT_YET_DEFINED | 1 | RW | Experimental OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Source 802.3. Configuration of the behaviour during the negotiation of the wire interface (master), which is sending a continuous stream of symbols for the remote site (slave) synchronizing its receiver on it |
| shortReachModeIsOn | Boolean false | 1 | RW | Experimental OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Activation of the Short Reach Mode for 10GBASE-T according to 802.3 45.2.1.64 |
| unidirectionalOperationIsOn | Boolean false | 1 | RW | Experimental OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Source 802.3. If (autoNegotiationIsOn=1) OR manualDuplexSelection=0 (=half duplex), this bit is ignored. When autoNegotiationIsOn=0 AND manualDuplexSelection=1 (=full duplex): 1 = Enable transmit from media independent interface regardless of whether the PHY has determined that a valid link has been established, 0 = Enable transmit from media independent interface only when the PHY has determined that a valid link has been established |
| wavelengthList | Integer -1 | 0..4 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: LENGTH_32_BIT - unit: pm - support: MANDATORY | Source: SFF-8690. Wavelength of the signal of laser in pico meter; multiplicity=0..3 for 10GBASE-LX4 according to 802.3 53.5 |
| _loopBackKindOn | LoopBackType NONE | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Source:802.3 according 22.2.4.1.2 Loopback. Maintenance Feature. The currently configured type of looping back of the wire interface header shall be expressed here. The received header is returned to the remote site. |
| isolationIsOn | Boolean false | 1 | RW | Experimental OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Source: 802.3. 1 = Activation of the separation of the PHY from higher network layers |
| restartPmdNegotiationIsOn | Boolean false | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Source: 802.3. Restarts the auto negotiation process |
| maintenanceTimer | Integer -1 | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: LENGTH_32_BIT - unit: s - support: MANDATORY | Time of existence of any maintenance configuration (e.g. the loop back). Valid values are defined in WireInterface::WireInterfaceCapability::maintenanceTimerRange Parameter and MW IM |
| problemKindSeverityList | WireInterfaceProblemSeverityType ./. | 6..* | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Severity of the problem to be configured. |

### WireInterfaceStatus

Qualified Name: WireModel::ObjectClasses::WireInterface::WireInterfaceStatus

Applied stereotypes:

- OpenModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- support: MANDATORY

Table 4: Attributes for WireInterfaceStatus

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| interfaceIsUp | Boolean false | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Source: 802.3 according 30.3.2.1.7 aPhyAdminState. 1 = A Physical layer entity (PHY) exists (including Medium Attachment Unit (e.g. SFP) ) and it is powered and can be managed |
| receiveSignalIsDetected | Boolean false | 1..11 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Source: 802.3 45.2.1.9 PMD receive signal detect.1 = Receiver (e.g. laser) detects signal; receiveSignalIsDetected[0]:total interface; receiveSignalIsDetected[1..10] different lanes of a multilane Medium Attachment Unit (MAU) |
| pmdIsUp | Boolean false | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Source: Inverse of 802.3 45.2.1.2.3 Fault (1.1.7). If (interfaceIsUp=1) BUT 0 = there is a fault in either transmit or receive path |
| pmdNegotiationState | pmdNegotiationStateType NOT_YET_DEFINED | 1 | R | Experimental OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Source: 802.3. Status of the Physical Medium Dependent (PMD) negotiation process (auto-neg) |
| _pmdCur | PmdType ./. | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Indicates the kind of Physical Medium Dependent (PMD) currently operated at this interface |
| _signalOrderingKindCur | SignalOrderingType NOT_YET_DEFINED | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Reference on a SignalOrderingType for expressing the currently active way of ordering the signals on the physical medium. |
| rxSyncRole | RxSyncRoleType NOT_YET_DEFINED | 1 | R | Experimental OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Indicates the result of the negotiation of the wire interface (master), which is sending a continuous stream of symbols for the remote site (slave) synchronizing its receiver on it |
| eeeIsUp | Boolean false | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | 1 = Energy Efficient Ethernet is supported at both ends of the link and it is activated |
| linkIsUp | Boolean false | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Parameter 1 = (transceiverIsUp=1) AND communication is established to the remote site |
| linkIsIdle | Boolean false | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | 1 = (linkIsUp=1) AND (eeeIsAvail=1) AND (eeeIsOn=1) AND link is currently in idle mode. If Energy Efficient Ethernet is not supported or switched off, this attribute must be 0. |
| txLevelCur | Integer 99 | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: false - valueRange: no range constraint - bitLength: LENGTH_8_BIT - unit: dBm - support: MANDATORY | Current transmit power |
| rxLevelCur | Integer 99 | 1..4 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: false - valueRange: no range constraint - bitLength: LENGTH_8_BIT - unit: dBm - support: MANDATORY | Current receive power; Also used for receive signal power measured at the Medium Dependent Interface (MDI) of 10GBASE-T during training as described in 802.3 55.4.3.1 |
| tempCur | Integer -99 | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: false - valueRange: no range constraint - bitLength: LENGTH_8_BIT - unit: Celsius - support: MANDATORY LikelyToChange | Current temperature (in degree Celsius) inside the transceiver To be moved to CoreModel::CorePhysicalModel-Initial::EquipmentDetail::ObjectClasses::DynamicDetails::PhysicalProperties |
| loopBackKindUp | LoopBackType NONE | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | The currently active (not just configured) type of looping back of the wire interface header shall be expressed here. The received header is returned to the remote site. Paramter and MW IM |

### WireInterfaceCurrentProblems

Qualified Name: WireModel::ObjectClasses::WireInterface::WireInterfaceCurrentProblems

Applied stereotypes:

- OpenModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- support: MANDATORY

Table 5: Attributes for WireInterfaceCurrentProblems

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| currentProblemList | WireInterfaceCurrentProblemType ./. | 0..* | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY |  |

### WireInterfaceCurrentPerformance

Qualified Name: WireModel::ObjectClasses::WireInterface::WireInterfaceCurrentPerformance

Aggregated performance information of the air interface at a particular moment.

Applied stereotypes:

- OpenModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- support: MANDATORY

Table 6: Attributes for WireInterfaceCurrentPerformance

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| currentPerformanceDataList | WireInterfaceCurrentPerformanceType ./. | 1..2 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | At least values of the counters, which are reset every 15 minutes, are to be provided. If available, the current values of the counters, which are reset every 24 hour, can be provided, too. |

### WireInterfaceHistoricalPerformances

Qualified Name: WireModel::ObjectClasses::WireInterface::WireInterfaceHistoricalPerformances

Aggregated performance information of the air interface for a pre-defined measurement interval.

Applied stereotypes:

- OpenModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- support: MANDATORY

Table 7: Attributes for WireInterfaceHistoricalPerformances

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| historicalPerformanceDataList | WireInterfaceHistoricalPerformanceType ./. | 0..* | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY |  |

## PureEthernetStructure_Pac

Figure 6: Exemplary PureEthernetStructure_Pac

Please, see chapter 15.2.1 in ONF TR-532v1.1 Microwave Information Model [ONF MW IM] for the exact definition of the PureEthernetStructure_Pac.

## EthernetContainer_Pac

Figure 7: Exemplary EthernetContainer_Pac

Please, see chapter 15.3.1 in ONF TR-532v1.1 Microwave Information Model [ONF MW IM] for the exact definition of the EthernetContainer_Pac.

## Data Types

### PmdType

Qualified Name: WireModel::TypeDefinitions::PmdType

Table 8: Attributes for PmdType

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| pmdName | String Name of the PMP not yet defined | 1 | R | OpenModelAttribute - partOfObjectKey: 1 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | The following PMD are not supported: '2BASE-TL', '10PASS-TS', '1000BASE-PX10-D', '1000BASE-PX10-U', '1000BASE-PX20-D', '1000BASE-PX20-U', '1000BASE-PX30-D', '1000BASE-PX30-U', '1000BASE-PX40-D', '1000BASE-PX40-U', '10GBASE-W', '10GBASE-EW', '10GBASE-LW', '10GBASE-SW', '10/1GBASE-PRX-D1', '10/1GBASE-PRX-D2', '10/1GBASE-PRX-D3', '10/1GBASE-PRX-D4', '10/1GBASE-PRX-U1', '10/1GBASE-PRX-U2', '10/1GBASE-PRX-U3' and '10/1GBASE-PRX-U4' To be chosen from the following list of PMD names according to IEEE 802.3 '10BASE5', 'FOIRL', '10BASE2', '10BROAD36', '10BASE-T', '10BASE-FP', '10BASE-FB', '10BASE-FL', '100BASE-T4', '100BASE-TX', '100BASE-BX10D', '100BASE-BX10U', '100BASE-FX', '100BASE-LX10', '100BASE-T2', '1000BASE-X', '1000BASE-BX10D', '1000BASE-BX10U', '1000BASE-LX', '1000BASE-LX10', '1000BASE-SX', '1000BASE-CX', '1000BASE-KX', '1000BASE-T', '10GBASE-X', '10GBASE-LX4', '10GBASE-KX4', '10GBASE-R', '10GBASE-ER', '10GBASE-LR', '10GBASE-SR', '10GBASE-LRM', '10GBASE-KR', '10GBASE-T', '10GBASE-PR-D1', '10GBASE-PR-D2', '10GBASE-PR-D3', '10GBASE-PR-D4', '10GBASE-PR-U1', '10GBASE-PR-U2', '10GBASE-PR-U3', '10GBASE-PR-U4', '40GBASE-R', '40GBASE-KR4', '40GBASE-CR4', '40GBASE-SR4', '4x10GBASE-SR', '40GBASE-LR4', '40GBASE-ER4', '40GBASE-FR', '100GBASE-R', '100GBASE-CR4', '100GBASE-KR4', '100GBASE-KP4', '100GBASE-CR10', '100GBASE-SR4', '100GBASE-SR10', '2x40GBASE-SR', '10x10GBASE-SR', '12x10GBASE-SR', '100GBASE-LR4', '100GBASE-ER4' and 'NOT_YET_DEFINED' |
| speed | String Speed of the PMP not yet defined | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: Mbit/s - support: MANDATORY | Line speed of the PMD. Value to be chosen from '2Mbit/s', '10Mbit/s', '100Mbit/s', '1000Mbit/s', '10Gbit/s', '40Gbit/s' or '100Gbit/s' |
| duplex | DuplexType NOT_YET_DEFINED | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | To be expressed, whether the PMD is full duplex or just half duplex |

### WireInterfaceProblemSeverityType

Qualified Name: WireModel::TypeDefinitions::WireInterfaceProblemSeverityType

Table 9: Attributes for WireInterfaceProblemSeverityType

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| problemKindName | String Problem kind name not defined. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Name of the alarm according to WireInterface::WireInterfaceCapability::supportedAlarms |
| problemKindSeverity | SeverityType WARNING | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: YES - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Severity of this type of alarm. |

### WireInterfaceCurrentProblemType

Qualified Name: WireModel::TypeDefinitions::WireInterfaceCurrentProblemType

Table 10: Attributes for WireInterfaceCurrentProblemType

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| problemName | String Problem name not specified. | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Name of the alarm according to supportedAlarmList attribute in MauType datatype |

### WireInterfacePerformanceType

Qualified Name: WireModel::TypeDefinitions::WireInterfacePerformanceType

Table 11: Attributes for WireInterfacePerformanceType

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| es | Integer -1 | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: LENGTH_32_BIT - unit: s - support: MANDATORY | Number of errored seconds according to 802.3 30.8.1.1.13 aLineESs |
| ses | Integer -1 | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: LENGTH_32_BIT - unit: s - support: MANDATORY | Number of severely errored seconds according to 802.3 30.8.1.1.12 aLineSESs |
| symbolErrorDuringCarrier | Integer -1 | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Source: 802.3 according to 30.3.2.1.5 aSymbolErrorDuringCarrier. Number of times when valid carrier was present and an invalid data symbol occured. |
| lowPowerIdleTransmitterMs | Integer -1 | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: ms - support: MANDATORY | Source: 802.3 similar to 30.3.2.1.8 aTransmitLPIMicroseconds. Number of milliseconds (original counter expresses microseconds), during which the transmitter was in power save mode |
| lowPowerIdleReceiverMs | Integer -1 | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: ms - support: MANDATORY | Source: 802.3 similar to 30.3.2.1.9 aReceiveLPIMicroseconds. Number of milliseconds (original counter expresses microseconds), during which the receiver was in power save mode |

### WireInterfaceCurrentPerformanceType

Qualified Name: WireModel::TypeDefinitions::WireInterfaceCurrentPerformanceType

Turns performance information into current performance information by inheriting from OTN_CurrentData.

Table 12: Attributes for WireInterfaceCurrentPerformanceType

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| performanceData | WireInterfacePerformanceType ./. | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: false - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY |  |

### WireInterfaceHistoricalPerformanceType

Qualified Name: WireModel::TypeDefinitions::WireInterfaceHistoricalPerformanceType

Turns performance information into historical performance information by inheriting from OTN_HistoryData.

Table 13: Attributes for WireInterfaceHistoricalPerformanceType

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| performanceData | WireInterfacePerformanceType ./. | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY |  |

## Enumeration Types

### LoopBackType

Qualified Name: WireModel::TypeDefinitions::LoopBackType

Parameter and MW IM

Contains Enumeration Literals:

- NONE:

- EXTERNAL:

  - Parameter

  - If the internal loopback test succeeds but the external loopback fails, the Medium Attachment Unit (MAU) is faulty (e.g. SFP has to be replaced)

- INTERNAL:

  - Parameter

  - If the internal loopback test fails, the Data Terminal Equipment (DTE) is faulty (e.g. board has to be replaced)

- NOT_YET_DEFINED:

### SeverityType

Qualified Name: WireModel::TypeDefinitions::SeverityType

MW IM

According to ITU-T M.3160

Contains Enumeration Literals:

- NON_ALARMED:

- WARNING:

- MINOR:

- MAJOR:

- CRITICAL:

### MdiKindType

Qualified Name: WireModel::TypeDefinitions::MdiKindType

Contains Enumeration Literals:

- SC:

  - Subscriber Connector

- FIBRE_CHANNEL_STYLE_1:

  - Copper connector

- FIBRE_CHANNEL_STYLE_2:

  - Copper connector

- BNC_TNC:

  - Bayonet/Threaded Neill-Concelman

- FC:

  - Fibre Channel coax headers

- FIBER_JACK:

- LC:

  - Lucent Connector

- MT_RJ:

  - Mechanical Transfer - Registered Jack

- MU:

  - Multiple Optical

- SG:

- OPTICAL_PIGTAIL:

- MPO1x12:

  - Multifiber Parallel Optic

- MPO2x16:

  - Multifiber Parallel Optic

- HSSDC_II:

  - High Speed Serial Data Connector

- COPPER_PIGTAIL:

- RJ45:

  - 8P8C, according to Clause 3 and Figures 1 through 5 of IEC 60603-7

- NO_SEPERABLE_CONNECTOR:

- MXC2x16:

- ST:

  - according to IEC 60874-10:1992, also often called BFOC/2.5

- NOT_YET_DEFINED:

### MediumKindType

Qualified Name: WireModel::TypeDefinitions::MediumKindType

Contains Enumeration Literals:

- TP_CAT3:

- TP_CAT5:

- TP_CAT6:

- TP_CAT8:

- SINGLE_MODE:

- MULTI_MODE:

- NOT_YET_DEFINED:

### MiiKindType

Qualified Name: WireModel::TypeDefinitions::MiiKindType

Contains Enumeration Literals:

- GBIC:

- SOLDERED_CONNECTOR:

- SFP_SFP+_SFP28:

- XBI_300_PIN:

- XENPAK:

- XFP:

- XFP_E:

- XPAK:

- X2:

- DWDM_SFP_SFP+:

- QSFP:

- QSFP+:

- CXP:

- SHIELDED_MINI_MULTILANE_HD_4X:

- SHIELDED_MINI_MULTILANE_HD_(X:

- QSFP28:

- CXP2:

- CDFP_STYLE1_STYLE2:

- SHIELDED_MINI_MULTILANE_HD_4X_FAN_OUT:

- SHIELDED_MINI_MULTILANE_HD_8X_FAN_OUT:

- CDFP_STYLE3:

- QSFP_MICRO:

- QSFP_DD:

- QSFP+_RATE_SELECT_V1:

- QSFP+_RATE_SELECT_V2:

- NOT_YET_DEFINED:

### DuplexType

Qualified Name: WireModel::TypeDefinitions::DuplexType

Contains Enumeration Literals:

- HALF_DUPLEX:

- FULL_DUPLEX:

- NOT_YET_DEFINED:

### SignalOrderingType

Qualified Name: WireModel::TypeDefinitions::SignalOrderingType

Contains Enumeration Literals:

- MDI:

- MDI_X:

- NOT_YET_DEFINED:

### RxSyncPreferenceType

Qualified Name: WireModel::TypeDefinitions::RxSyncPreferenceType

- Experimental

Contains Enumeration Literals:

- AUTO_MULTIPORT_DEVICE:

  - Master/Slave relation is determined by auto-negotiation and this device has a preference for being Master

- AUTO_SINGLE_PORT_DEVICE:

  - Master/Slave relation is determined by auto-negotiation and this device has a preference for being Slave

- MANUAL_MASTER:

  - Master/Slave relation is determined by manual configuration and this device shall be Master

- MANUAL_SLAVE:

  - Master/Slave relation is determined by manual configuration and this device shall be Slave

- NOT_YET_DEFINED:

### RxSyncRoleType

Qualified Name: WireModel::TypeDefinitions::RxSyncRoleType

- Experimental

Contains Enumeration Literals:

- MASTER:

- SLAVE:

- FAULT:

- NOT_YET_DEFINED:

### pmdNegotiationStateType

Qualified Name: WireModel::TypeDefinitions::pmdNegotiationStateType

- Experimental

Contains Enumeration Literals:

- NONE:

  - Device does not support auto negotiation

- DISABLED:

  - Auto negotiation is disabled.

- ENABLED:

  - Auto negotiation is enabled, but currently nothing is connected

- IN_PROGRESS:

  - The auto-negotiation protocol is running and negotiation is currently in-progress

- COMPLETE:

  - The auto-negotation protocol has completed successfully

- FAILED:

  - The auto-negotation protocol has failed

- NOT_YET_DEFINED:

  - Device supports autonegotiation, but state is currently unknown, e.g. because device just booted

## Super Classes

### WireCurrentProblem

Qualified Name: WireModel::ObjectClasses::SuperClasses::WireCurrentProblem

Applied stereotypes:

- OpenModelClass

- objectCreationNotification: NO

- objectDeletionNotification: NO

- support: MANDATORY

Table 14: Attributes for WireCurrentProblem

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| sequenceNumber | Integer ./. | 1 | R | OpenModelAttribute - partOfObjectKey: 1 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Unique sequence number of the current problem object. |
| timeStamp | DateAndTime 20101120140000.0Z+1 | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Time and date of the problem. _format:yyyyMMddhhmmss.s[Z¦{+¦-}HHMm]; yyyy='0000'..'9999' year; MM='01'..'12' month; dd='01'..'31' day; hh='00'..'23' hour; mm='00'..'59' minute; ss='00'..'59' second; s='.0'..'.9'tenth of second (set to '.0' if EMS or NE cannot support this granularity); Z='Z' indicates UTC (rather than local time); {+¦-}='+' or '-' delta from UTC; HH='00'..'23' time zone difference in hours; Mm='00'..'59' time zone difference in minutes. |
| problemSeverity | SeverityType WARNING | 1 | R | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Severity of the alarm. |

## Notifications

### AttributeValueChangedNotification

Qualified Name: WireModel::Notifications::AttributeValueChangedNotification

To be sent when the value of an attribute has changed.

Applied stereotypes:

- OpenModelNotification

- triggerConditionList: invalid

- support: MANDATORY

Table 15: Attributes for AttributeValueChangedNotification

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| counter | Integer -1 | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: LENGTH_32_BIT - unit: no unit defined - support: MANDATORY | Counts attribute value changed notifications. |
| timeStamp | DateAndTime 20101120140000.0Z+1 | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY |  |
| objectIdRef | UniversalId ./. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | ID of the affected *_Pac. |
| attributeName | String Attribute name not specified. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Name of the attribute that has been changed. |
| newValue | String New value not specified. | 0..1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Attribute value converted to a string (xml, json, ...) |

### ObjectCreationNotification

Qualified Name: WireModel::Notifications::ObjectCreationNotification

To be sent when a new *_Pac has been instancieted.

Applied stereotypes:

- OpenModelNotification

- triggerConditionList: invalid

- support: MANDATORY

Table 16: Attributes for ObjectCreationNotification

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| counter | Integer -1 | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: LENGTH_32_BIT - unit: no unit defined - support: MANDATORY | Counts object creation notifications. |
| timeStamp | DateAndTime 20101120140000.0Z+1 | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY |  |
| objectIdRef | UniversalId ./. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | ID of the affected *_Pac. |
| objectType | String Type of created object not specified. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Type of Object to be chosen from the following list of values: 'WireInterface_Pac', 'MW_PureEthernetStructure_Pac' or 'MW_EthernetContainer_Pac'. |

### ObjectDeletionNotification

Qualified Name: WireModel::Notifications::ObjectDeletionNotification

To be sent when an instance of a *_Pac has been deleted.

Applied stereotypes:

- OpenModelNotification

- triggerConditionList: invalid

- support: MANDATORY

Table 17: Attributes for ObjectDeletionNotification

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| counter | Integer -1 | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: LENGTH_32_BIT - unit: no unit defined - support: MANDATORY | Counts object deletion notifications. |
| timeStamp | DateAndTime 20101120140000.0Z+1 | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY |  |
| objectIdRef | UniversalId ./. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | ID of the affected *_Pac. |

### ProblemNotification

Qualified Name: WireModel::Notifications::ProblemNotification

To be sent when a problem occured.

Applied stereotypes:

- OpenModelNotification

- triggerConditionList: invalid

- support: MANDATORY

Table 18: Attributes for ProblemNotification

| Attribute Name | Type DefaultValue | Multiplicity | Access | Stereotypes | Description |
| --- | --- | --- | --- | --- | --- |
| counter | Integer -1 | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: LENGTH_32_BIT - unit: no unit defined - support: MANDATORY | Counts problem notifications |
| timeStamp | DateAndTime 20101120140000.0Z+1 | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY |  |
| objectIdRef | UniversalId ./. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | ID of the affected *_Pac. |
| problem | String Problem name not specified. | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Name of the problem according to WireInterface::WireInterfaceCapability::supportedAlarmList. |
| severity | SeverityType WARNING | 1 | RW | OpenModelAttribute - partOfObjectKey: 0 - AVC: NO - isInvariant: true - valueRange: no range constraint - bitLength: NA - unit: no unit defined - support: MANDATORY | Severity of the problem according to WireInterface::wireInterfaceConfiguration::problemSeverityList |

# References

| Reference | Comment |
| --- | --- |
| [IEEE802.3] | IEEE 802.3-2018 IEEE Standard for Ethernet, June 2018 (https://standards.ieee.org/standard/802_3-2018.html) |
| [ONF Core IM] | TR-512 ONF Core Information Model Version 1.2, September 2016 (https://www.opennetworking.org/images/stories/downloads/sdn-resources/technical-reports/TR-512_CIM_(CoreModel)_1.2.zip) |
| [ONF UMGL] | TR-514 UML Modeling Guidelines; Version 1.2, September 2016 (https://www.opennetworking.org/images/stories/downloads/sdn-resources/technical-reports/TR-514_UML_Modeling_Guidelines_v1.2.pdf and https://www.opennetworking.org/images/stories/downloads/sdn-resources/technical-reports/IISOMI_514_UML_Modeling_Guidelines_v1.2.pdf) |

# Back matter

## Editors

Thorsten Heinze

Telefonica Germany GmbH & Co. OHG

Martin Skorupski

highstreet technologies GmbH

## Contributors

Many thanks to Michael Binder, Giorgio Cazzaniga, George Clapp, Petr Jurcik, Danilo Pala, James Ries, Daniela Spreafico, Alexandru Stancu, Beatriz Ugrinovic, Tracy Van Brakle, Yossi Victor, Yang Yonggang, Bernd Zeuner and Nader Zein for reviewing and commenting of the Ethernet PHY Model.

_______________________